Checklist of E/OHS Activities for Confined Space Entry

Program Contact Person: 	Roger Voelz

Is the Confined Space Entry Plan in place?  Yes  No N/A

Is the Plan current?  Yes  No N/A

Has the Plan been reviewed this school year?  Yes  No N/A

Have confined space areas been identified?  Yes  No N/A

Have measurements to include CCI/ft been completed?  Yes No N/A

Are permit entry forms in place?  Yes  No N/A

Location:  Activities Manual

Are confined space labels in the proper locations?  Yes  No N/A

Is a list of employees eligible to enter confined spaces complete?  Yes  No N/A

Has training for affected employees completed?  Yes No

Date of completion: 9/25/2013

Confined Space Inventory

Building: Cedar Mt. High School

Building Contact: Mike Siemer 	Program Contact: Brian Parrie

Completed By: _Brian Parrie   Date: 9/25/2013

| Room Identification Name Given Space | Opening Dimension | # Dimension Of Confined Space | ## Potential Hazards | ## Permit/ Non- Permit | Labeled | Photo ID # |
| --- | --- | --- | --- | --- | --- | --- |
| Boiler Room | 44”X47” | 48”X48”X 200’ | Asbestos, heat electric | Permit required | No | See AM |
| Tunnels |  |  |  |  |  |  |
| Boiler Room | 20” diameter | 13’X4’X 5’ | LE, O2 deficient., heat | Permit required | Yes |  |
| Boilers |  |  |  |  |  |  |
| Gym Lobby | 4”X2” | 8’X3’X4’ | LE, Asbestos, heat | Permit required | Yes |  |
| Air handler |  |  |  |  |  |  |
| Boiler Room | 16”round | 45”X9’ | Engulfment, heat | Permit Required | Yes |  |
| Water Tank |  |  |  |  |  |  |
| Boiler room | 45”X35” | 45”X35”X>200’ | LE, Asbestos, heat, electrical | Permit Required | No |  |
| Tunnel |  |  |  |  |  |  |
| Various Areas | 7’ high X 2’ wide | 18’X20”X11’6” high | LE, | Non-Permit Required | No |  |
| Pipe Chases |  |  |  |  |  |  |
| Elevator | 7’X3” | 6”x5’X4” | Entrapment, electrical, hydraulic pressure | Permit | Yes |  |
| Pit |  |  |  |  |  |  |