Checklist of E/OHS Activities for Confined Space Entry

Program Contact Person:  Jo Pyle

Is the Confined Space Entry Plan in place?  Yes  No N/A

Is the Plan current?  Yes  No N/A

Has the Plan been reviewed this school year?  Yes  No N/A

Have confined space areas been identified?  Yes  No N/A

Have measurements to include CCI/ft been completed?  Yes No N/A

Are permit entry forms in place?  Yes  No N/A

Location:  Activities Manual

Are confined space labels in the proper locations?  Yes  No N/A

Is a list of employees eligible to enter confined spaces complete?  Yes  No N/A

Has training for affected employees completed?  Yes No

Date of completion: 4/1/13

Note: Confined space entry activities at this school are limited to licensed contractors only. School staff does not enter confined space areas.

Confined Space Inventory

Building: Tracy High School

Building Contact: Jo Pyle	Program Contact: Brian Parrie

Date: 2-14-13

| Room Identification Name Given Space | Opening Dimension | # Dimension Of Confined Space | ## Potential Hazards | ## Permit/ Non-Permit/Alternate | Labeled | Photo ID # |
| --- | --- | --- | --- | --- | --- | --- |
| Boiler room | 18” | 7’X3’X3.5’ | Ele.O2, Heat | Permit | Yes |  |
| Boiler #1 |  |  |  |  |  |  |
| Boiler Room | 18” | 7’X3’X3.5’ | Ele.O2, Heat | Permit | Yes |  |
| Boiler #2 |  |  |  |  |  |  |
| Boiler Room | 4’X15’ | 100’X15’X4’ | Heat, electricity | Non-permit | Yes |  |
| Tunnel |  |  |  |  |  |  |
| (2)Chimney stacks | 18”X18” | 30” round X 24’ high | Particulate matter, heat | Permit | No |  |

Building: Tracy Elementary School

Building Contact: Jo Pyle	Program Contact: Brian Parrie

Date: 2-14-13

| Room Identification Name Given Space | Opening Dimension | # Dimension Of Confined Space | ## Potential Hazards | ## Permit/ Non-Permit/Alternate | Labeled | Photo ID # |
| --- | --- | --- | --- | --- | --- | --- |
| Boiler room | 18” | 7’X3’X3.5’ | Ele.O2, Heat | Permit | Yes |  |
| Boiler #1 |  |  |  |  |  |  |
| Boiler Room | 18” | 7’X3’X3.5’ | Ele.O2, Heat | Permit | Yes |  |
| Boiler #2 |  |  |  |  |  |  |

Note: View entry points with this link Confined Space Photos.